Automotive

Engineering Body Materials

(Past, Present and Future Analysis)

Steel Becomes the “Material of Choice”

for Auto Bodies

•	Original bodies made of wood

& steel

•	Quick to tool, easy to change

•	Large number of skilled workers

Benz Patent Motorwagen 1886 (Replica)

•	1914 Dodge & Budd produce

5,000 all-steel bodies

•	All-steel body was lighter, stronger, easier to make

•	Most significant advantage was in painting

Dodge Brothers Touring Car 1917

“Every time the price of steel goes up,

auto makers moan in anguish and intensify their search for

replacements”

gas

“Every time the price of steel goes up, auto makers moan in anguish and intensify their search for

replacements”

Materials Causing the Biggest Stir in Body

Technology

Plastics

-“been getting the big play …

as successor to steel”

Aluminum Alloys

-“about as strong as steel, less than half the weight”

-“used …since birth of auto”

-“most plentiful metal on earth”

Magnesium Alloys

-“even lighter than aluminum, stronger for its weight than steel”

-“most easily machined and cast”

-“enough in the ocean…

to provide 100,000,000 tons a year”

Published October 1953

“The day of the passenger car made primarily of iron and steel is on the wane! Some sources predict that

by 1960 a Cadillac will weigh less than a 1953 Chevrolet

and a Chevy will probably weigh about as much as a motorcycle.”

Prediction	50+	Years	Ago…

Steel

Mg

Al	Plastics

PAST

FUTURE

Key Product Drivers & Resulting Vehicles

Clean Air Act

Highway Safety Act

Global

Customer

Safety

Focus

Oil

Shock

Competition

Personalization

1950

1960

1970	1980

1990	2000+

1997199720062006–– AudiAudi–– TheTheintroduceintroduceC6C6 Z06Z06

19841984 –– PontiacPontiac releasesreleases

ththee  allall aluminumaluminumCorvetteCorvette volumeisvolumeis released.released.

FieroFiero.. Small,Small, fuelfuel

efficientefficientcar.car.sporsporUtilizesUtilizesTheThetstsAluminumAluminumcarcarcarboncarbonthatthat	frfiberfiber

usesusesweighweighcompositecompositefenders,fenders,uupp totopanelspanelslesslesswheelwheelthanthanhouses,houses,

toto reducereducesteelsteelfloors,floors,ovovfrframeseameserralalll andandweweofofiightghtcompanan..   aluminumaluminum

design.design.frame.frame.

Late 1970’s – Dodge releases

Charger XL that applies higher

ces thethe dy.

es allall

strength steel and aluminum components that resulted in a

ss thethe firstfirst

roduction 19531953 –– ChevroletChevrolet Introdu

ames	CorvetteCorvette ww// ffiberglassiberglass bo

40%40% ooff	LightweightLightweight bodybody imprimprov arable	aspectsaspects ofof perfperformance.ormance.

285kg reduction in vehicle mass.

Steel Mass Reduction

0%

10%

MILD STEELS

CONV HSS

20%

30%

40%

50%

Mass Reduction

Opportunity

1970

1980

1990	2000	2010	2020

Time

2030

Current Body and Closure

Metallic Material Content

Advanced

HSS

Aluminum & Magnesium

Source: Ducker Worldwide

2007

Conventional

HSS

12.7%

9.5%	0.8%

Medium

HSS

15.8%

Mild Steel

54.6%

Bake Hardenable Steel

6.6%

850 Pounds

Comparison of Materials by Performance

|  | Density |  | Modulus | Tensile Strength | Elongation | Corrosion Resistance |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mild Steel |  |  |  |  |  |  |  |
|  |  | - - - - - - - - - - BASELINE - - - - - - - - - - |  |  |  |  |  |
| Aluminum | + |  | - | 0 | 0 | 0 / + |  |
| Magnesium | + |  | - - | 0 | - | 0 / - |  |
| Polymers & Composites | + |  | 0 / - | - / 0 / + | - | + |  |
| HSS/AHSS | 0 |  | 0 | + + | 0 / - | 0 |  |

Comparison of Materials by Manufacturability

Steel &

HSS/AHSS

Aluminum

Magnesium

Polymer/ Composites

Advantages:

•	Formability

•	Weldability

•	Infrastructure

•	Painted Surface

Disadvantages:

•	Castability

Advantages:

•	Formability

•	Castability

•	Painted Surface

Disadvantages:

•	Lower formability than steel

•	Weldability

Advantages:

•	Castability

Disadvantages:

•	Formability

•	Elevated temperature stamping & hemming

Advantages:

•	Low cost tooling

•	Shorter lead time

Disadvantages:

•	Cycle Time

•	Infrastructure

•	Difficult to repair

•	Painted

•	Weldability

Surface

Comparison of Materials by

Other Important Characteristics

|  | Cost (Est. $/lb.) | Availability (Annual Metric Ton Production) | Environment (Primary Production Emissions reported by industry) |
| --- | --- | --- | --- |
| Mild Steel (Baseline) | $0.50 | 110 Million | • easy to recycle • Emissions = 2.3 – 2.7 kg CO2/kg |
| Aluminum (Al) | $1.00 | 2.5 Million | • easy to recycle • Emissions = 13.9 – 15.5 kg CO2/kg |
| Magnesium (Mg) | $1.50 | 0.5 Million | • easy to recycle • Emissions = 18.0 – 24.8 kg CO2/kg |
| PMC | $1.20 - 6.50 | Unknown | • difficult to recycle • Emissions = 2.5 – 23.0 kg CO2/kg |
| HSS/AHSS | $0.55 | Included in Mild Steel | • easy to recycle • Emissions = 2.3 – 2.7 kg CO2/kg |

Steel Strategy

70

60	Second Generation AHSS

IF- HS

ISO

50	Conventional HSS

40

Mild

30	BH

20

10

0

First Generation AHSS

MART

0	300

600

900

1200

1600

Elongation (%)

Tensile Strength (MPa)

Business	Strategy

Manufacturability

& Cost:

• 	material

• 	design

• 	manufacturing

Government Regulations

• 	Fuel Economy

• 	Crash Performance

• 	Emissions

& Customer Requirements

• 	Cost

• 	Quality/Styling

• 	Features

Steel Mass Reduction Opportunities

0%

10%

20%

30%

MILD STEELS

CONV HSS

Without DOE Support

AHSS

40%

50%

With Steel Strategy

& DOE Support

FreedomCAR GOAL

Technology

Decision

MASS COMPOUNDING

3rd GENERATION

Mass Reduction

Opportunity

1970

1980

1990	2000	2010	2020

Time

2030

V eh icles p er T h o u sa n d P eo p le

800

Global	Transportation Growth

Source: Joe Carpenter, DOE

U.S.

600

400

Industrialized Pacific

Western Europe

200

0

Africa

China

Eastern Europe

Mid-East

Former Soviet Union

Latin America

Developing Asia

U.S. Historic

1900	1910	1920	1930	1940	1950	1960	1970	1980	1990

China, with 13 vehicles per 1000 people, is where the U.S. was in 1913

China’s population is currently 4 times the population of the U.S.

What materials will be used

to meet these demands?

NORTH AMERICAN

LIGHT VEHICLE METALLIC MATERIAL TRENDS

Body and Closure Metallic Material Content by Type

2007 	2015

Con. HSS

Advanced

HSS

9.5%

12.7%

Aluminum & Magnesium

0.8%

Advanced

HSS

34.8%

Aluminum & Magnesium

2.5%

Mild Steel

29.0%

15.8%

Medium

HSS

6.6%

54.6%

Mild Steel

10.2%

23.5%

Bake Hardenable

Conventional

HSS

Bake Hardenable and

Medium HSS

850 Pounds

Source: Ducker Worldwide

800 Pounds with an Equal

Footprint to 2007

NORTH AMERICAN

LIGHT VEHICLE METALLIC MATERIAL TRENDS

North American Light Vehicle Material Content Per in Pounds

|  | 1975 | 2005 | 2007 | 2015 | Change From 1975 to 2015 |
| --- | --- | --- | --- | --- | --- |
| Mild Steel | 2,180 | 1,751 | 1,748 | 1,314 | Down 866 lbs. |
| High Strength Steel | 140 | 324 | 334 | 315 | Up 175 lbs. |
| Advanced HSS | -- | 111 | 149 | 403 | Up 403 lbs. |
| Other Steels | 65 | 76 | 76 | 77 | Up 12 lbs. |
| Iron | 585 | 290 | 284 | 244 | Down 341 lbs. |
| Aluminum | 84 | 307 | 327 | 374 | Up 290 lbs. |
| Magnesium | -- | 9 | 9 | 22 | Up 22 lbs. |
| Other Metals | 120 | 150 | 149 | 145 | Up 25 lbs. |
| Plastic/Composites | 180 | 335 | 340 | 364 | Up 184 lbs. |
| Other Materials | 546 | 629 | 634 | 650 | Up 104 lbs. |
| Total Pounds | 3,900 | 3,982 | 4,050 | 3,908* | Up 8 lbs. |

Source: Ducker Worldwide

* Same vehicle mix and average footprint as 2007

NORTH AMERICAN

LIGHT VEHICLE METALLIC MATERIAL TRENDS

Body and Closure Metallic Material Content by Type

Advanced

HSS

34.8%

North America

Aluminum & Magnesium

2.5%

Mild Steel

29.0%

PHS, TWIP

& Others

12.0%

Europe

Aluminum & Magnesium

3.0%

Mild Steel & BH

25.0%

10.2%

Conventional

HSS

23.5%

Bake Hardenable and

Medium HSS

30.0%

AHSS

30.0%

Conventional HSS

2015

Why is this reasonable?

•	No real changes in basic trends over past 50 years

•	“Cash is King” – customer paying less than before & demanding more

•	Infrastructure not ready

•	Availability – Infrastructure of metal production

•	Global Differences in Needs

What could	‘disrupt’ this	prediction

•	Fuel Cost & Availability

•	Material Cost & Availability

•	Economic Stability

•	Government Regulations

•	Technological Discoveries/Advances

Conclusions

•	Always be a need/desire to push to lightweight materials for the auto industry

•	Doesn’t mean no steel

•	Current prediction is mainly some form of steel

– Best value to customer (performance/cost)

•	However, disruptive event(s) is as likely, or even more likely, than ever before